### Operating Instructions 1-10DE99

Wear Parts

Section 7

  - Wear Parts

7.2	Maintenance Log

### Operating Instructions 1-10DE99

Wear Parts

7.1 Wear Parts

Certain portions of the blast unit are continuously exposed to high velocity abrasive.  These areas and their corresponding parts must be periodically inspected for wear and erosion to keep from damaging components that are not normally exposed to the abrasive blast.  These parts are identified on the service drawings later in this manual along with part numbers.  Maintenance personnel design all parts for relatively easy replacement.  A preventive maintenance program should be performed to monitor these areas on a regular basis.  The following list outlines these areas and the time intervals involved.

# Wear Parts Chart

| Part | Inspection Interval | Wear Indication |
| --- | --- | --- |
| Blast Wheel | 5 Hrs. | Blades Worn Half Thickness or More |
| Feed Spout | 50 Hrs. | Thin at Wheel Entry |
| Control Cage | 5 Hrs. | Eroded Edges |
| Rebound Plenum | 100 Hrs. | Thin Portions; Wear on Welds |
| Deflector Plate | 50 Hrs. | Thin Sections; Wear at Welds |
| Separator | 100 Hrs. | Thin Sections; Wear at Welds; Warpage; Screen Wear |
| Liners | 50 to 75 Hrs. | Thin Sections; Warpage |
| Blast Wheel Hub | Every Other Change of Blast Wheel | Pins Missing or Worn; Wear |
| Top Cover | 50 Hrs. | Inside Edge Erosion |
| Blast Housing | When Changing Liners. | Liners Worn Through; Corners Eroded |

Keep in mind that other portions of the machine require periodic inspections and maintenance.  The parts mentioned above are associated with the internal abrasive blast.  Electrical cable and exhaust hose, for example, also require close inspection and maintenance.

### Operating Instructions 1-10DE99

Wear Parts

7.2 Maintenance Log

## MAINTENANCE LOG

# Electric Equipment-

| Liners – Inspect for wear | Checked |  | OK |  | Change |  |
| --- | --- | --- | --- | --- | --- | --- |
| Blastwheel/cage - Inspect for wear | Checked |  | OK |  | Change |  |
| ### Filters - Inspect – clean or replace | Checked |  | OK |  | Change |  |
| ### Airline - Inspect for leaks | Checked |  | OK |  | Change |  |
| Shot valve – Inspect | Checked |  | OK |  | Change |  |
| Seals – Inspect for wear | Checked |  | OK |  | Change |  |
| Bearings – Inspect set screws and grease | Checked |  | OK |  | Change |  |
| ### Inspect power cord for damage | Checked |  | OK |  | Change |  |
| ### Magnets & Seals – Inspect for wear | ### Checked |  | OK |  | Change |  |
| Traction Drive – Inspect chain & sprockets working smoothly |  |  | OK |  | Change |  |

#### Visit us on the web @ www.blastrac.com