### Operating Instructions 1-10DE99

Specifications

Section 12

  - Blast Unit – Specifications

  - Dust Collector – Specifications

  - Site and Equipment Electrical Requirements

### Operating Instructions 1-10DE99

Specifications

12.1 Blast Unit Specifications

| Table 3: 1-10DE99 - MECHANICAL DATA |  |
| --- | --- |
| Length | 5 feet |
| Width | 1 foot 4 inches |
| Height | 3 feet 6 inches |
| Weight | 425 pounds |
| Wheel | 6-1/2 inches |
| Cleaning Width | 10 inches |
| Bearing Unit: Operating RPM Grease | 6800 RPM Mobilux 77 EP-2 or Mobilelith AW-2 or equivalent |

| ### Table 4: Blast Unit Motor Data |  |  |  |
| --- | --- | --- | --- |
| Motor | Horsepower | FLA @ 208 | FLA @ 230 |
| DC Gearmotor | 1/4 HP | 3.5* | 3.5* |
| 1-10DE Blast | 10 HP | 27.6 | 25.0 |
| *The DC Gearmotor Drive Is 120V. It derives power from the control power transformer. |  |  |  |

12.2 Dust Collector Specifications

| Table 1: 4-54 MECHANICAL DATA |  |
| --- | --- |
| Length | 4 feet |
| Width | 2 feet |
| Height | 5 feet |
| Weight | 345 pounds |
| Filter Cartridges | 4 (6 inches - diameter; 16 inches in length) |
| Total Filter Area | 216 square feet |
| Air Flow | 400 CFM |

### Operating Instructions 1-10DE99

Specifications

12.2 Cont’d

| ###### Table 2: 4-54 MOTOR DATA |  |  |
| --- | --- | --- |
| Motor | Horsepower | FLA @ 208/230 |
| Compressor | 3/4 HP | 2.4 |
| Blower | 2.0 HP | 5.8 |

12.3 Site and Equipment Electrical Requirements

## Site and Equipment Electrical Requirements for

## Dust Collector/Blast Unit Combinations

| ##### Table 8: Site Power Requirements and Blast Motor Maximum Blast Amps |  |
| --- | --- |
| Parameter | ##### Dust Collector/Blast Unit Combinations |
|  | 4-54/1-10DE |
| Minimum Ampere Circuit @ 208/230V | 50 AMP |
| Maximum Ampere Circuit @ 208/230V | 100 AMP |
| Minimum Generator Size | 30 kW |
| Maximum Blast FLA @ 208/230 | 25.0 AMP |